CA-162W B&W BULLET CAMERA - 1/3” CCD - 420 TV Line - 0.02 Lux - 4mm Lens - 12V DC

Image Sensor; 1/3” Interline CCD. Effective Pixels; 512(H) x 492(V). H. Resolution; 420 TV line. Min. Illumination; 0.02 Lux, F:2.0. Synchronizing System; Internal. Scanning System; 525 line, 60 field/sec. Video Output; 1.0Vp-p composite, 75Ω. S/N Ratio; More than 48 dB (AGC off). Electronic Shutter; 1/60~100,000 sec. Gamma Correction; r=0.45. Lens; 3.6mm fixed lens. Power Source; DC 12V (± 10%). Operation Temperature; -40°C~50°C RH 95% Max. Preservation Temperature; -40°C~60°C RH 95% Max. Power Consumption; 100mA. Dimensions; 0.9Ø x 2 inch. Case Material; Aluminum. *Above specifications and design subject to change without notification