LENS-04 4MM LENS - Focal Length: 3.94mm - F-Number: 2.5 - Sensor Size: 1/3” CCD - Back Focal Length: 5.92 In Air - Flange Back Length: 17.526 In Air - Elements: 4 Glass

Focal Length; 3.94mm. F-Number; 2.5. Sensor Size; 1/3” CCD. Operation; Iris; Fixed. Focus; 10cm ~ ∞. Angle of View; Diagonal; 1/3”, 94°. Horizontal; 1/3”, 74°. Vertical; 1/3”, 54°. Entrance Pupil Diameter (mm); 1.58. Entrance Pupil Position (mm); 3.92. Exit Pupil Diameter (mm); 6.28. Exit Pupil Position (mm); -15.59. Back Focal Length (mm); 5.92 In Air. Flange Back Length (mm); 17.526 In Air. Mount Type; C mount. Dimensions (inch); 1.4 x 1. *Above specifications and design subject to change without notification