MTR/LCD19 19” LCD MONITOR - 19 Inch Viewable SXGA(1280x1024) LCD - Brightness 280 cd/㎡ - Contrast Ratio 500 : 1 (Typical) - Response Time 8ms - Analog Interface - Built-in power supply - Power Management (VESA DPMS) - Plug-and-Play (VESA DDC2B Compatibility) - TCO'03 Certified - Support VESA Physical Mounting Interface

Size; 19 inch. Type; A-Si TFT Active Matrix. Viewing Angle (H/V); 150/130(CR≥10). Pixel Pitch (H/V); 0.294 mm (H x V). Resolution; 1280 x 1024. Contrast Ratio; 500 : 1. Luminance; 280 cd/㎡. Active Display Size (H x V); 376.32 x 301.056 mm. Display Color; 16.2 Mil. Video; R.G.B Analog. Sync; H/V - Separate. Connector; 15 pin D-sub. Horizontal; 30kHz ~ 80 kHz. Vertical; 60~75Hz. Max. Resolution (Analog); 1280 x 1024 @ 75Hz. Analog; 720 x 400 @ 70 Hz, 640 x 480 @ 60/72/75 Hz, 800x600 @ 56/60/72/75 Hz, 1024 x 768 @ 60/70/75 Hz, 1280 x 1024 @ 60/75 Hz. VESA Arm; 7.5 x 7.5 ㎝. Audio system; 2W x 2ch / Earphone Jack. Stand; Tilt Stand. VESA Arm; 7.5 x 7.5 ㎝. User Control; Brightness, Contrast, H/V-Position, Auto, phase, Color temperature selection, OSD location adjustment, Language, Reset, Exit. Power Source; 100 ~ 240V AC 1.0A. Power Consumption; 60W (Max.). Safety & Regulations; FCC, CE, CB, VCCI, UL/CUL, CCC, PSE, GOST,TUV-GS, C-Tick, NOM, BSMI, EK, MIC. Unit Dimensions & Weight; 16.9 x 17 x 7.8 inch, 10.8 lbs. *Above specifications and design subject to change without notification