SK-2005 B&W MINI SQUARE CAMERA - 1/3” Sony CCD - Compact Size (1.2” x 1.2”) - Video, audio, and power cable included - 3.7mm Lens - 12V DC - Audio Output - Built-In Microphone

Image Sensor; 1/3” B&W CCD. Effective Pixels; 510(H) x 492(V). H. Resolution; 400 TV line. Min. Illumination; 0.5 Lux. Audio Gain; 40dB±2dB. Audio Max Output; 600 Ohm Max. 3Vp-p. Frequency Response; 300Hz~20,000Hz. Audio S/N Ratio; -45dB±10%. Synchronizing System; Internal. Scanning System; 2:1 interlace. Video Output; 1.0Vp-p (sync. Negative) termination 75Ω. S/N Ratio; More than 45dB or more (AGC off). Gain Control; Automatic. Electronic Shutter; 1/60~100,000 sec. Gamma Correction; r=0.45. Lens; 4.3mm fixed lens, f/2.0. Power Source; DC 12V. Power Consumption; Max 110mA. Reverse Polarity Protection; Yes. Humidity; RH 95% Max. Preservation Temperature; -40°C~60°C RH 95% Max. Operation Temperature; -10°C~50°C. Dimensions; 1.2 x 1.2 inch. Case Material; Aluminum. *Above specifications and design subject to change without notification