VPD-IR024HVA Color Vandal Proof IR Dome Camera - 1/3” Sony Super HAD CCD - 600 TV lines - 24 IR LEDs - 4 ~ 9mm Varifocal Lens - 12V DC - Easy camera adjustment

Image Sensor; 1/3" Sony Super HAD CCD. Effective Pixels; 811H × 508V. Resolution; 600 TV-Lines. Shutter Speed; 1/60 ~ 1/100,000sec.. Iris Control; Auto. Gamma Characteristic; r=0.45. S/N Ratio; 45dB. Sync. System; Negative Sync., Internal. Min. Illumination; 0.1 Lux (CDs on: 0 lux). Video Output; 1.0 Vp-p Composite Video (75Ω load). Lens; 4mm~9mm Vari-Focal. Backlight Compensation; Auto. AGC; Auto. Power Supply; DC12V ±10%. Power Consumption; 450mA Max (DC12V). Operating Temp.; -10℃ ~ +50℃. Operating Humidity; 90% RH max.. Dimensions; 4.8(Ø) x 4(H) inch (approx.). Weight; 1.8 lbs (approx.). *Above specifications and product design subject to change without notifications