VPD-IR024HVM Vandal Proof IR Dome Camera 2.8~12mm Manual Iris Varifocal Lens - 1/3” Sony Super HAD CCD - 650 TV lines - 24 IR LEDs - 2.8~12mm Varifocal Lens - 12V DC - Easy camera adjustment

Image Sensor; 1/3" Sony Super HAD CCD. Effective Pixels; 811H × 508V. Total Pixels; 768H x 494V. Scanning System; 2:1 Interlaced 525 TV Lines. Scanning Frequency; 15,734KHz (H) / 59.94Hz. Shutter Speed; 1/60 ~ 1/100,000sec.. Sync. System; Internal. Resolution; 650 TV-Lines. White Balance; ATW. Backlight Compensation; On / Off. Flickerless; On / Off. AGC; On / Off. Video Output; 1.0 Vp-p Composite Video (75Ω load). S/N Ratio; 45dB. Min. Illumination; 0 Lux for Infrared Lighting Conditions. IR Range; 60ft. Iris Control; Auto. LED Quantity; 24 pcs. Gamma Characteristic; r=0.45. Lens; 2.8mm~12mm Vari-Focal. Operating Temp.; -10℃ ~ +50℃. Power Consumption; DC 12V 110mA (CDS On: 420mA). Dimensions; 4.8(Ø) x 3.9(H) Inch (approx.). Weight; 1.8 lbs (approx.). *Above specifications and design subject to change without notice